USAGE of MUDP_Log_DataExtracter.exe

Below is the command format to run MUDP_Log_DataExtracter.exe 
this tool takes XML and .txt file as input arguments.

1.Extractor Tool

MUDP_Log_DataExtractor.exe MUDP_DATA_Extracterv1p2.xml  f_list.txt

To convert Delphi Vehicle Log format (*.dvsu,*.mf4,*.vpcap,*.pcap)  into csv 
Create a logs.txt file, and add the (*.dvsu,*.mf4) path into the text file
enable options in the XML and run the Exe .

- USAGE:

- To extract the streams in the CSV format , configure the XML with the below tags
-     Set CSV_MODE to “ 1”

- select the SENSOR_STATUS as per requirement

- select OUTPUT_STREAM_OPTION  with respect to requirement.

- To extract the XML trace file,  configure the XML with the below

-    Set XML_TRACE_MODE and CSV_MODE  to “ 1” 
 -   To Get all 4 Radar Sensor ‘s XML Traces  Enable all 4 radar sensors from SENSOR_STATUS and enable all Z-streams from Radar_Stream_Option and set the output_xml_trace_option depends on requirement.

-   To Get BPIL Sensor ‘s XML Traces  Enable BPIL  sensors from SENSOR_STATUS and enable all Z-streams from Radar_Stream_Option and set the output_xml_trace_option depends on requirement.

-   To Get ECU XML Traces  Enable RADAR_ECU  from SENSOR_STATUS and enable ECU0, ECU1, ECU3 from RADAR_ECU_Stream_Option and set the output_xml_trace_option depends on requirement.

2.Packet_Loss Tool

MUDP_Log_DataExtracter.exe Packet_loss_v1p0.xml f_list.txt

To convert Delphi Vehicle Log format (*.dvsu,*.mf4,*.vpcap,*.pcap)  into csv 
Create a logs.txt file, and add the (*.dvsu,*.mf4) path into the text file
enable options in the XML and run the Exe .

1.To extract the packet loss text file, , configure the XML with the below

-    Set Packet_Loss_Report to “ 1”

- To print both error info + Packets loss statistics set PACKET_LOSS_STATISTICS to “1”
 -   Disable OUTPUT_STREAM_OPTION & RADAR_ECU_STREAM_OPTION

-   Select SENSOR_STATUS with respect to requirement.

2. If there is no packet los for the given log it will just print Report as Pass if it there are any packet loss it will give the information about that particular sensor.

Design updates in this tool:
1.  Renamed Radar_dump_tool to MUDP Log Data Extracter  tool . 
2.  Previously this tool takes cmd line arguments , Currently updated the tool to take XML and file list as input parameters . 
3.  This tool will create CSVs and Report 
4.   Report file contains Scan Index  information  (like Total number of scan index dropped , % of Scan   Index dropped etc) 
5.  This tool is able to run on following file inputs - Single DVSU,ORCAS MDF, Canape MDF, VIGEM MDF
6.  This tool is updated for all srr5 customers .
7.  MUDP_DATA_Extracter_V1p2.XML is created to enable / disable generation of components .
    Following options are available in the XML

- To generate csv for all stream enable  options CDC, Z7A Core,Z7B Core, Z4 Core, Z7BCustomer , Z4 Customer at a time.
 -  XML have option to save output at Same location as input / Separate output file path.
 -  XML have option to extract data from all radars / choose each radar position 
8.  MUDP_DATA_Extracterv1p2.xml is used to extract csv and also xml traces.

9. This tool support wireshark logs such as PCAP and VPCAP.

10. Tool generates xml traces with detections , tracks etc information

for both ecu and sensors

11. Packet Loss Tool works for both sequential and continuous mode

12. All stream can be extracted one time